******************************************************************************

********************* INSTALL.DOC (Version 3.4.0, = 4.0 DOS) *****************

******************************************************************************

Directory of this file:

1) Program-BAT files

2) Installation path

3) Installation files

4) Configuration files

5) Print files

6) List of supported plotters, printers and graphic files

7) List of direct supported cryo systems

8) Installation manually

9) Program start problems

10) Text files

11) Hardware and DOS configuration

12) Software protection

13) Changing configuration after installation

14) Format of data files

******************************************************************************

*                     Program-BAT files                                      *

******************************************************************************

The program BAT-files are in the subdirectory \DLTS\DLTFS\BAT.

The normally call for the DLTFS program is DL. Some further calls with DL are:

DL SIM   : use of only the simulation program, no hardware necesarray

DL NOCRYO: start without cryo system

DL NOPERI: start without plotter/printer

DL OLD   : start your old DLTFS version

You get further options by call DL ?. The options for calling DL are also

described in the file \DLTS\DLTFS\HARDCONF\BAT.DAT.

Special configuration can make in the files \DLTS\DLTFS\HARDCONF\PROG_X.DAT.

Then you have to call DL DX, where X is a number from 1 until 7.

If you have an old and a new program version installed (after updating),

you can use the old version with the call DL OLD.

List of BAT- or EXE-files:

1) DL      : Normal start of the DLTFS program. The command line options

are descripted in PROGRAMS.DOC.

2) DLSET   : Sets the main configuration, change the default addresses,

makes user pathes, list the PC-, IEEE-, option- and hardlock key-

configuration (similiar to the INSTALL program), make new text

files or copies the files from the bug fix disk. Without command

line parameter, you can select the action in the DLSET program.

The main command line options are:

TEXT --> Make new text files

INFO --> List PC, IEEE, options and hardlock key configuration

PORT --> Search all ports for the hardlock key

For further options call DLSET ?.

3) DLASC   : Convert a binary file to ASCII format. You can work with

wildcards (?,*) for conversion of many files. The origin binary

files are overwritten. The ASCII-file has a header of 80 lines.

Files of edit plot program (first extension X or W) are always

in his own ASCII format (header of 20 lines). With the option /C

you get a ASCII table file without the header. For more

information type DLASC ? or see in the file DATA.DOC.

4) DLCOPY  : This program copy (update-) files from disk, exctract files

from an ASCII e-mail file or concact files to an e-mail file.

5) DLPATH  : Not a true program, only an utility for other BAT files.

This file jump to \DLTS\DLTFS.

6) TEST_DSK: Print disk number, program version and license number on the

screen. Without parameter the drive A will be selected. In

case of drive B type TEST_DSK B. This program is in the

subdirectory INSTALL.

7) LIBTOASC: Convert the library data bank file in an ASCII or TEX file.

This file is in the subdirectory \DLTS\LIB.

******************************************************************************

*                     Installation path                                      *

******************************************************************************

SubDirectories from \DLTS

DLTFS    : DLTFS-program

OLDDLTFS : your old version of the DLTFS-Program (after updating)

WORK     : main work directory

LIB      : DBASE library file and library tools

DATA     : directory for common measurement data

USER_X   : directory for data or for data- and init-path of USER_X,

if installed

SubDirectories from \DLTS\DLTFS :

BAT            : Program-Bat-Files [*.BAT] and programs [*.EXE]

INSTALL        : Installation file

INSTALL\DEFAULT: Default-files

HARDCONF       : Hardware-Configuration [*.DAT], special configuration [*.CON]

HARDCONF\CRYO  : Files for cryo system [*.DAT]

HARDCONF\MEAS  : Files for measuring system [*.DAT]

INIT           : User-Init-Files [*.I*], default

INITDAT        : Program-Init-Files [*.DAT, *.BIN, *.68K, *.DBF, *.DBT]

RUNDAT         : Data-Files [*.DAT, *.CHR, *.BIN]

TEXT           : Text-Files at Program-Start [*.LGE, *.LEN]

TEXT\MAKETEXT  : Source-Text-Files             [*.LGE, *.LEN]

TEXT\INFOTEXT  : Text-Files for infos          [*.LGE, *.LEN]

PRG            : Main program [*.EXE], Overlays [*.OVL],

PRG\DRV        : Resident-Overlays [*.OVL] (Graphic, IEC, Bridge, RS232, ...)

PRG\PRIV       : Private overlays and programs (no file at install-time)

DOC            : Documentation (*.DOC) and report (*.REP) files

READBUFF       : Read-Buffer-Files [*.0*]

SubDirectories from \DLTS\WORK :

INIT           : User-Init-Files [*.I*, *.CON]

FIGURES        : User figure files [*.B*]

BUFFER         : Read-Write-Buffer-Files (no file at install-time)

PMD            : Post-mortem-dump files (no file at install-time)

The main directory is \DLTS\DLTFS. In the following the descripted

directories are subdirectories of \DLTS\DLTFS.

******************************************************************************

*                     Installation files                                     *

******************************************************************************

BAT\DLPATH.BAT            : BAT-File, change to DLTFS drive and dir

BAT\DL.BAT                : BAT-File for DLTFS program

HARDCONF\HARDWARE.DAT     : Hardware-Configuration

HARDCONF\PERI_*.DAT       : Printer and Plotter

HARDCONF\BAT.DAT          : Definitions of Cmd-Line-Parameter

HARDCONF\CRYO\CRYO_*.DAT  : Cryo-System

HARDCONF\MEAS\FAST_*.DAT  : Fast-pulse generator

\DLTS\WORK\INIT\COLORS.IEA: Screen-Parameter

******************************************************************************

*                     Configuration files                                    *

******************************************************************************

All global hardware parameters are on the directory HARDCONF\MEAS. You should

not change these files. You find an explaination of the parameters in each

file. The program and user configuration paramters are on the directory INIT.

For further information have a look in the file INIT\CMD.IZA.

******************************************************************************

*                             Print files                                    *

******************************************************************************

If you have not the configuration manual or if you have not the last version

of this manual, you can print this manual by a text printer, because the

files are ASCII files. The following give you a list of files, which you

should print.

Subdirectory INSTALL\DEFAULT:

1) CONFIG.SYS   : Example for DOS

2) AUTOEXEC.BAT : Example for DOS

Subdirectory HARDCONF:

1) HARDWARE.DAT     : Hardware-Configuration

2) PERI_1.DAT       : Printer and Plotter

3) CRYO\CRYO_31.DAT : Cryo-System

4) MEAS\FAST_1.DAT  : Fast-pulse generator

Subdirectory INIT:

1) CMD.IZA      : Names of init files, overview of init files

2) COLORS.IEA   : Colors of screen

3) PLOT.IFA     : Parameter for plots (screen and plotter)

4) PRINT.IGA    : Parameter for plotter and printer

5) USERCONF.IHA : User configuration

6) GLOBALS.IIA  : Global parameters

7) PROGLOW.IKA  : LowLevel parameter

8) MATERIAL.IMA : Material parameter (effective mass, ...)

9) SAMPLE.IOA   : Sample parameter (header file)

Subdirectory DOC:

1) INSTALL.DOC  : Installation, DOS configuration, software protection

2) DATA.DOC     : Data structure

3) LOAD.DOC     : how to load a private overlay or program

4) VERSIONS.DOC : List of current file versions

******************************************************************************

*       List of supported plotters, printers and graphic files               *

******************************************************************************

Type of plotters:

0: unknown

1: HP7470, A4 with 6 pens, automatic set of terminator and number of pens

2: HP7475, A3 with 6 pens, automatic set of terminator and number of pens

3: DraftPro, A4 with 8 pens, automatic set of terminator and number of pens

4: DraftPro, A3 with 8 pens, automatic set of terminator and number of pens

5: as printer DeskJet 1200

6: as printer DeskJet 1600

Type of printers:

1: only text or PS, monochrom

2: only text or PS, color

3: PCL 3, no HPGL, monochrom: DeskJet 500, 510, 520

4: PCL 3, no HPGL, color    : DeskJet 550C, 560

5: PCL 4, no HPGL, monochrom: LaserJet II

6: PCL 4, no HPGL, color    :

7: PCL 5+, HPGL, monochrom  : LaserJet III, IV, V

8: PCL 5+, HPGL, color      : DeskJet 1200, 1600, PaintJet

9: PCL 5+, HPGL, color      : as plotter, DeskJet 1200

Type of graphic/data files:

1) ASCII x-y

2) ASCII x-y1...yn

3) HPGL/PLT plotter file, monochrom and color

4) HPGL printer (LaserJet) file, monochrom and color

5) PCL file, monochrom

6) HiRes PCL file, monochrom

7) PCX file, monochrom, color 4 Bit, color 8 Bit

8) BMP file, monochrom, color 4 Bit, color 8 Bit

9) HiRes PCX file, monochrom

10) ACAD DXF file, monochrom

11) EPS PostScript file, monochrom, color

******************************************************************************

*               List of direct supported cryo systems                        *

******************************************************************************

Specifig types of cryo systems with CRYO_*.DAT-files (file/type-number):

- / 0: no cryo system, simulation

- / 1: no cryo system, only room temperature

- / 2: no cryo system, room and N2 temperature

12 /12: external Keithley resistance meter and Pt-table

21/ 21: external EXE-file

22 /22: old Cryophysics-He (as DRC91/old but with waiting time)

23 /23: temperature controller T-3000

31 /31: Cryophysics-He DRC91/old, old Biorad-He

32 /32: Eurotherm-RS232

33 /33: Oxford ITC503

34 /34: temperature controller 9650

35 /35: Cryophysics-He DRC93/91

36 /36: Oxford ITC502,ITC4

41 /41: Biorad-N2 old

51 /51: Biorad-N2 new

52 /52: Biorad-He new

53 /53: AME

54 /54: Chinon

55 /55: Linkam controller

******************************************************************************

*                     Installation manually                                  *

******************************************************************************

1) AUTOEXEC.BAT : PATH ....\DLTS\DLTFS\BAT

CONFIG.SYS   : a) FILES=20

b) Ramdisc-Driver, if you use a ramdisc for faster working

c) 512KB EMS memory for more memory

d) Cache-Program for faster working

Use not many resident programs. The DLTFS program needs more than 550KB

(better 600KB) free memory. Use on a 80386-Computer a Memory-Manager.

An example for AUTOEXEC.BAT and CONFIG.SYS you find in the subdirectory

\DLTS\DLTFS\INSTALL\DEFAULT.

2) Set the drive, where the DLTFS-program is stored

--> \DLTS\DLTFS\BAT\DLPATH.BAT

A change is only necesarry, if the drive is not C

3) Use or not use of a ramdisc

--> \DLTS\DLTFS\HARDCONF\HARDWARE.DAT

4) Address of IEEE-card of the computer, or IEEE-card not present

--> \DLTS\DLTFS\HARDCONF\HARDWARE.DAT

In line 40 ist the value for the address. IF there is no IEEE-card of the

computer, set the address to 0. This is only possible for the simulation

program.

5) Default-choice of your cryosystem

--> \DLTS\DLTFS\HARDCONF\HARDWARE.DAT

Line 110 for the type of your cyrosystem. See in the section before or in

the file DOC\CRYO.DOC for types and further information.

6) IEEE-Address or RS232-Address of your cryosystem

--> \DLTS\DLTFS\HARDCONF\CRYO\CRYO_X.DAT

X ist the type number of your cryosystem. In line 3 is the address. For

your type see at 5) or in the file DOC\CRYO.DOC

7) Parameters of the cyrosystem or specially cryosystems

--> \DLTS\DLTFS\HARDCONF\CRYO\CRYO_X.DAT

X is the type number of your cryosystem.

8) IEEE-Address OR RS232-Address of your plotter, or no plotter

--> \DLTS\DLTFS\HARDCONF\PERI_1.DAT

Line 3 for the address.

Address 0 for no plotter

Address 1..31 for IEEE

Address 211 for the first Com-Port (com1), 212 for the second Com-Port.

For other adresses and RS232-protocoll have a look in this file.

9) Default-language, Default-User-Init-Names

--> \DLTS\DLTFS\HARDCONF\HARDWARE.DAT

10) Screen-Colors

--> \DLTS\DLTFS\INIT\COLORS.IEA

11) Capacitance of cables or cryosystem

--> \DLTS\DLTFS\HARDCONF\MEAS\CALIB.DAT

In line 10 is the additional cable capacitance. This correction is only

necessarry if this capacitance can not be corrected on the capcitance

bridge.

******************************************************************************

*                    Program start problems                                  *

******************************************************************************

If you have problems at starting of the DLTFS program, start the program

followings: DL TEST. In this case you get a list of founded or not founded

files on the screen and in the file DOC\START.REP.

IF you start with DL OFF the program works without ramdisc, EMS,... .

IF you start with DL IGNORE the errors at start will be ignored. Be carefull

with this option!

You can combine the options.

If an error appears, the error messages are always stored in the file

WORK\STARTERR.REP.

Please start the DLTFS-program from the command line and not from the Norton

commander or such program, because memory ploblems!

******************************************************************************

*                          Text files                                        *

******************************************************************************

You can manipulate the Text-Files with an ASCII-Editor. Use "cr" and "lf" for

the end of a line. You find those files in \DLTS\DLTFS\TEXT,

\DLTS\DLTFS\TEXT\INFOTEXT, \DLTS\DLTFS\TEXT\MAKETEXT. Files with the extension

LEN are for the english version, those with LGE for the german version.

After manipulation you must call the program DLSET TEXT. The BAT-File for this

program is in \DLTS\DLTFS\BAT.

******************************************************************************

*                 Hardware and DOS configuration                             *

******************************************************************************

1) MS-DOS-AT-Computer with 80286 CPU or higher

2) 80287 or higher coprocessor

3) VGA graphic card

4) IEEE card from Keythley (not necessary for the simulation program)

5) Minimal 550 KByte free memory

6) Start the DLTFS-program from the command line and not from the Norton

commander or such program, because memory ploblems!

7) DOS 3.2 or higher.

8) Helpfull: DOS 5.0 or higher

9) Helpfull: Load all resident programs in the high memory.

10) Helpfull: Ramdisk with 1.5 MByte

11) Helpfull: Memory for a cache program, like SMARTDRV (min. 1 MByte)

12) Helpfull: 600 KByte or more free memory (80386-computer, DOS 5.0)

13) Helpfull: 512 KByte EMS memory (80386-computer, DOS 5.0)

14) ITS measurements and evaluations need more than 600 KByte free memory

and EMS memory of 256 kByte until 512 KByte. Do not work with ITS without

EMS! You get memory and speed problems without EMS. ITS need also high

computer performance like a 80486 processor.

15) For the use of a mouse you need a mouse driver. Please use a mouse

driver which uses only a small part of memory. In case of memory

problems please dont install the mouse driver and dont work with

the mouse.

16) Set a path to the subdirectory \DLTS\DLTFS\BAT.

17) You find an example for the CONFIG.SYS and the AUTOEXEC-BAT file

in the subdirectory \DLTS\DLTFS\INSTALL\DEFAULT.

******************************************************************************

*                        Software protection                                 *

******************************************************************************

For the use of this software you need a hardlock key! You must

connect this hardlock key with the parallel port (printer port)

of your computer. Instead of a hardlock key, you can use a integrated

hardlock card for your computer. The disks and the hardlock key must have

the same license number.

Make sure that the hardlock key is correct installed before calling the

DLTFS-program. If you have connected a printer on the same parallel port

as the hardlock key, perhaps you must switch on the printer. In some cases

the hardlock key dont work on the parallel port of a combi controller.

Please use here an additional parallel port card.

The DLTFS program use two kinds of hardlock keys, one for the

measurement program, the other for the pure simulation program.

The hardlock key for simulation is marked with the red letters SIM.

At the installation with the program INSTALL you must decide which

program you want to install. If you have made a wrong installation

you can correct it in line 65 in the file HARDWARE.DAT.

You find this file in the subdirectory \DLTS\DLTFS\HARDCONF.

For the measurement program there must be a N, for the pure simulation

program a Y. Only this flag in line 65 defines which hardlock key

is necessary for the installed DLTFS program. The calling parameter SIM

at the command line has no effect of the selection of the hardlock key.

If you have installed the measurement program you need for working

the hardlock key for measurement. If you want to start only the simulation

program you must then call DL SIM. You can not start a DLTFS program which

was installed for the measurement hardlock key with the simulation

hardlock key, also if you want only to simulate. On the other hand you need

for the pure simulation program the hardlock key with the letters SIM.

******************************************************************************

*               Changing configuration after installation                    *

******************************************************************************

For changing the parameters of configuration after installation please use the

program DLSET. You can call it from every path.

******************************************************************************

*                     Format of data files                                   *

******************************************************************************

You find a description of the data files, made by the user in the DLTFS-

program, in the file DOC\DATA.DOC.