******************************************************************************

***************************** PC_CONF.DOC ************************************

******************************************************************************

Directory of this file:

1) DOS configuration

2) Software protection

3) Multi-DOS-configuration

4) Example of the CONFIG.SYS file for DOS

5) Example of the AUTOEXEC.BAT file for DOS

******************************************************************************

*                     DOS configuration                                      *

******************************************************************************

Use so many memory as possible for the DLTFS-program. The free DOS memory

size must be bigger than 550 KByte, better is 600 KByte and more.

Please use additionally 256 KByte EMS memory or more. Without EMS or with

to small DOS memory you can get memory problems, escpecially in the

isothermal program, the tempscan maximum evaluation and the manual

plot program. Load, if possible, all resident DOS programs in the high memory.

Therefore please use DOS 5.0 or higher. The program works also under WINDOWS

and OS 2 in a DOS box. For faster working install a ramdisk of 1.5 MByte and

a cache program like SMARTDRV. An example for the CONFIG.SYS file you find in

the next section. IF you have a printer and you want to work in the high

resolution PCL mode, you need 512 KByte until 1 MByte EMS memory. The size

depend on the size of the plots.

Start the DLTFS program from the command line and not from the Norton

commander or such programs because memory ploblems.

For ITS measurements there exist special conditions. For a good work here

you need more than 600 KByte DOS memory and 256 KByte EMS memory.

Without EMS you get memory problems and the program works much slower.

ITS evaluations should only be done on a computer with a 80386

or higher processor, bcause the calculation takes a long time.

Please turn on the flag for the 80386 processor.

For the use of a mouse you need a special mouse driver. Please use,

if possible, a mouse and a driver for three buttons.

The left button is the main button. It works as mark and enter. The mid

button marks and the right button works as enter key. Marking means in the

menu to mark the menu point, in the plot to set the marker. Enter means to

take over the marked menu point resp. the current marker position (not mouse

position).

******************************************************************************

*                     Software protection                                    *

******************************************************************************

For the use of this software you need a hardlock key! You must

connect this hardlock key with the parallel port (printer port)

of your computer. Instead of a hardlock key, you can use a integrated

hardlock card for your computer. The disks and the hardlock key must have

the same license number.

Make sure that the hardlock key is correct installed before calling the

DLTFS-program. You can test it with DLSET INFO. If you have connected a

printer on the same parallel port as the hardlock key, perhaps you must

switch on the printer. In some cases the hardlock key dont work on the

parallel port of a combi controller. Please use here an additional parallel

port card.

The DLTFS program use two kinds of hardlock keys, one for the measurement

program, the other for the pure simulation program. The hardlock key for

simulation is marked with the red letters SIM. At the installation with the

program INSTALL you must decide which program you want to install. The calling

parameter SIM at the command line has no effect of the selection of the

hardlock key. If you have made a wrong installation you can correct it with

DLSET.

If you have installed the measurement program you need for working

the hardlock key for measurement. If you want to start only the simulation

program you must then call DL SIM. You can not start a DLTFS program which

was installed for the measurement hardlock key with the simulation

hardlock key, also if you want only to simulate. On the other hand you need

for the pure simulation program the hardlock key with the letters SIM.

******************************************************************************

*                     Multi-DOS-configuration                                *

******************************************************************************

The DLTFS program needs many memory, so that you have to load all resident

programs in the high memory. In special cases here is not enough memory,

so that these programs use the conventional memory. If you have the computer

not only for the DLTFS program and you need for the other work many resident

programs or you want to work with WINDOWS, the best solution is to make

different configurations of the CONFIG.SYS and the AUTOEXEC.BAT file.

In the DLTFS installation program there is a possibility to say make a

preperation of multi DOS configuration. In this case there will be made

a directory BOOT and copied some standard files to this directory.

You find a description there in the file DOSCONF.DOC. This file is also

in the directory \DLTS\DLTFS\INSTALL\BOOT.

******************************************************************************

*                 Example for the CONFIG.SYS file of DOS                     *

******************************************************************************

rem Example for a 80386 computer with 4 MByte memory

rem The path "DOS" in the following lines depends on your DOS-configuration

BREAK=ON

COUNTRY=1,,C:\DOS\COUNTRY.SYS

rem The value 1 in the line before depends on your country

BUFFERS=10

FILES=20

rem Necessary for multi tempscan files

DEVICE=C:\DOS\HIMEM.SYS

rem DEVICE=C:\DOS\EMM386.EXE x=CC00-CDFF ram 512

DEVICE=C:\DOS\EMM386.EXE x=CC00-CDFF ram 512 frame=D800

rem DEVICE=C:\DOS\EMM386.EXE x=CC00-CDFF ram 512 frame=E000

rem The value C000-CDFF depends on your IEEE-card

rem Without IEEE card delete the part x=...

rem The EMS-memory of 256 kB is helpfull, and necessary for ITS measurements

rem The EMS-memory of 512 kB is necesarry for high resolution PCL prints

rem In the most cases you can switch the frame address from D000 (without

rem frame parameter) to E000. Then you have 64KB more memory space. Please

rem use this option! Only in some cases, e.g. for a PCI board, the frame

rem address of E000 gives address conflicts with the PC hardware. Then in

rem the most cases you can set the frame address to D800. However the only

rem total sure address is D000, but here you lost 32 or 64KB memory.

DOS=HIGH,UMB

rem DEVICEHIGH=C:\DOS\MOUSE.SYS

rem DEVICEHIGH=C:\DOS\GMOUSE.SYS/1 1

rem For the mouse support you need a special mouse driver

rem Dont use a mouse driver which needs many memory

DEVICEHIGH=C:\DOS\RAMDRIVE.SYS 1536 512 64 /e

SHELL=COMMAND.COM /E:512 /P

rem Load, if possible, all resident programs in the high memory, so that

rem you have more than 600 kB free memory. This is very important

rem especially for ITS measurements.

******************************************************************************

*                 Example for the AUTOEXEC.BAT file of DOS                   *

******************************************************************************

@ECHO OFF

C:\DOS\SMARTDRV.EXE /X

LOADHIGH C:\DOS\KEYB UK,,C:\DOS\KEYBOARD.SYS

LOADHIGH C:\DOS\DOSKEY

rem LOADHIGH C:\DOS\PRINT /D:PRN

rem This line is necesary if you want to work with the DOS spooler

LOADHIGH C:\DOS\MODE LPT1:,,P

rem This line is helpfull if you want to work with a printer

PATH C:\DOS;C:\USERCOMM;C:\DLTS\DLTFS\BAT;

PROMPT $p$g