******************************************************************************

********************* PROGRAMS.DOC (Version 3.4.0) ***************************

******************************************************************************

In this file are all DLTFS programs will be explained. Except of TEST_DSK

and LIBTOASC all files are in the subdirectory \DLTS\DLTFS\BAT, so that you can

start these programs from every directory. If you call these programs with the

option ?, for example DL ?, the program dont start but gives a list of options

and an explaination for the syntax.

The following programs exist:

1) DL      : Normal start of the DLTFS program. The command line options

are descripted in the next chapter.

2) DLSET   : Sets the main configuration, change the default addresses,

makes user pathes, list the PC-, IEEE-, option- and hardlock key-

configuration (similiar to the INSTALL program), make new text

files or copies the files from the bug fix disk. Without command

line parameter, you can select the action in the DLSET program.

The main command line options are:

TEXT --> Make new text files

INFO --> List PC, IEEE, options and hardlock key configuration

PORT --> Search all ports for the hardlock key

For further options call DLSET ?.

3) DLASC   : Convert a binary file to ASCII format. You can work with

wildcards (?,*) for conversion of many files. The origin binary

files are overwritten. The ASCII-file has a header of 80 lines.

Files of edit plot program (first extension X or W) are always

in his own ASCII format (header of 20 lines). With the option /C

you get a ASCII table file without the header. For more

information type DLASC ? or see in the file DATA.DOC.

4) DLCOPY  : This program copy (update-) files from disk, exctract files

from an ASCII e-mail file or concact files to an e-mail file.

5) DLPATH  : Not a true program, only an utility for other BAT files.

This file jump to \DLTS\DLTFS.

6) TEST_DSK: Print disk number, program version and license number on the

screen. Without parameter the drive A will be selected. In

case of drive B type TEST_DSK B. This program is in the

subdirectory INSTALL.

7) LIBTOASC: Convert the library data bank file in an ASCII or TEX file.

This file is in the subdirectory \DLTS\LIB.

******************************************************************************

*                Command line options for the DL program                     *

******************************************************************************

ENG    : Use of the english text

GER    : Use of the german text

PLOT   : Call for the external presentation plot program

EDIT   : Call for the external edit plot program

NOCRYO : No use of cryo system, no measurement of temperature

NOPERI : Switch off the use of a plotter or printer

SIM    : Simulation program, no measurement possible, works without hardware

IEEE   : Jump to the IEEE command menu and additional as IGNORE

TEST   : Give a report of founded and not founded files at start on the screen

and in the file REP\START.REP

IGNORE : Ignore errors at start. Be carefull with this option!

LOCAL  : User only local instead of network program

OFF    : Switch off use of ramdisk, EMS, ...

NORAM  : Use no ramdisk, although it is installed

MEAS   : Measuring task for OS2 or WINDOWS

EVAL   : Evaluation and simulation task for OS2 or WINDOWS

OLD    : Loads the old program version