3.4.0

23.04.1997

Directory

1. Keys in the menu, window and input

2. Keys at the page editor and at listing of data

3. Marker

4. Mouse

5. Plot programs

6. Structure of the main plot programs

7. Others

1.1  Keys in the menu

0 : Jump back to the menu before

1..9 : Jump to the menu point

X : Jump out of a main modul (program)

ALT-1 .. ALT-9 : Jump to the first menu level, only possible in modules

with a beam in the header

ALT-F1..ALT-F9 : Jump to the global functions (not always possible)

F1 .. F9 : Jump to the special funtion of this modul (not always

possible)

Cursor keys : Cursor movement

ENTER : Start action or go to next menu

ESC  : Leave the menu, jump back to the menu before

HOME : Home position of menu

ALT-A : Select standard or alternative user class. This switch can

be selected permanently or local. In the second case switch

the program after a jump back from a menu to the normal user

class

ALT-M : Shows the biggest free memory space

ALT-P : Change of private user class

ALT-F11 : Screen tools (hardcopy)

Function and ALT-function keys:

ALT-F1 : Explaination of the menu point or jump to the global

information menu, if at home position

ALT-F1,ALT-F1 : Two times pressed in a short time gives from all positions

the information menu

ALT-F2 : Global menu for directory, files and operationg system

ALT-F3 : Menu for sample parameters

ALT-F4 : Menu for globals parameters

ALT-F5 : Control of cryo system

ALT-F6 : Menu for bridge

ALT-F7 : Menu for CC-DLTS

ALT-F9 : Menu for calculator

ALT-F11 : Screen tools (hardcopy)

F3 : List data

F4 : Standard plot

F5 : Save data

F6 : Read data

1.2  Keys in the window input

Cursor keys : Cursor movement

ENTER : Start action

ESC : Break of the action

HOME : Home position of window

ALT-F1 : Explaination of the window point or global help of the

window, if at home position

ALT-A : Select standard or alternative user class. Work not in all

cases

ALT-P : Change of private user class

ALT-F11 : Hardcopy

1.3  Keys in the value input

0..9,-,+ : Integer values

0..9,-,+,e,E : Real values

Characters : Text

CursLeft : Cursor movement to left

CursRight : Cursor movement to right

CursUp : Cursor movement to the first character

CursDown : Cursor movement to the last character

HOME : Cursor movement to the first character

END : Cursor movement to the last character

ENTER : Take over value

ESC : Break

BACKSPACE : Delete a character and goes left

DEL : Delete a charcater

INS : Insert a character

ALT-INS : Insert a character with repeat function

PageDown : Delete all characters right from the cursor position

ALT-F1 : Help

CTRL-R : Restore the old input

For graphic plots and especially for HPGL plots are additional keys possible:

CTRL-E : Exponent character

CTRL-I : Index character

CTRL-D : Draw a symbol defined by the next character:

1:square, 2:cross, 3:caro, 4:x, 5:triangle, 6:full square

(not HPGL), 7:full square, 8:circle, 9:full circle, A:point,

B,C,D: line with length of 1,2,3 characters

CTRL-K : Select pen with number defined by the next character

(only HPGL)

CTRL-L : Set cursor one position to left (only HPGL)

CTRL-F : Swap between the standard font and the alternate font,

defined in PRINT.IGA. Normally the alternate font is a

mathematic font. After a string the program switch to the

normal font. The swapping of font is only in the HPGL mode

for printers (LaserJet, PaintJet, DeskJet1200) possible.

2.1  Keys at the page editor

Cursor keys : Cursor movement

ENTER : Go to the next input

ESC : Break of the action

BACKSPACE : Delete a character and goes left

DEL : Delete a character

INS : Insert a character

ALT-INS : Insert a character with repeat function

CTRL-DEL : Delete a line

CTRL-INS : Insert a line

PageDown : Go to the next page

PageUp : Go to the page before

F1 : Opens an input window, only in some cases

F5 : Take over inputs and end

2.2  Keys at listing of data

ENTER : List the next page

ESC : Break of action

PageDown : List the next page

PageUp : Go to the page before

CursDown : Increment of the array index

CursUp : Decrement of the array index

SPACE : Go to the next page at page data or list the next line

at line data

F7 : Print data or save data to an ASCII file

A11 : Screen tools (hardcopy)

3.1  Vertical marker

CursLeft : Cursor movement to left, slow

CursRight : Cursor movement to right, slow

CursUp : Cursor movement to left, fast

CursDown : Cursor movement to right, fast

CTRL-CursLeft : Cursor movement to left, fast

CTRL-CursRight : Cursor movement to right, fast

CTRL-CursUp : Cursor movement to left, very fast

CTRL-CursDown : Cursor movement to right, very fast

ALT-CursLeft : Cursor movement to left, fast

ALT-CursRight : Cursor movement to right, fast

ALT-CursUp : Cursor movement to left, very fast

ALT-CursDown : Cursor movement to right, very fast

ENTER : Take over value

ESC : Break

3.2  Index marker

CursLeft : Cursor movement to left, slow

CursRight : Cursor movement to right, slow

CursUp : Cursor movement to left, fast

CursDown : Cursor movement to right, fast

ENTER : Take over value

ESC : Break

3.3  Cross marker

CursLeft : Cursor movement to left, slow

CursRight : Cursor movement to right, slow

CursUp : Cursor movement to up, slow

CursDown : Cursor movement to down, slow

CTRL-CursLeft : Cursor movement to left, fast

CTRL-CursRight : Cursor movement to right, fast

CTRL-CursUp : Cursor movement to up, fast

CTRL-CursDown : Cursor movement to down, fast

ALT-CursLeft : Cursor movement to left, fast

ALT-CursRight : Cursor movement to right, fast

ALT-CursUp : Cursor movement to up, fast

ALT-CursDown : Cursor movement to down, fast

ENTER : Take over value

ESC : Break

4. Mouse

If the mouse arrow is visible:

Left button : Marks the point and take over or start

Mid button : Marks the point

Right button : Take over or start, same meaning as ENTER

If you press first the mid button and then the right button, it is the same

meaning as to press only left button.

If you have a mouse with only two buttons, you get the function of the mid

button if you press left and right button together.

IF the mouse arrow is not visible then only the right button is valid and

has the same mening as ENTER.

You can also call the function key actions by the mouse. For this you have

to klick at the function key symbol or the fuction key text. At informations

in the status line (last line) you can klick in this line or in the line

before. At plots with brackets like <StdPrg> you get the full plot

possibilities if you klick above of <StdPrg>.

5.1  Standard plot program <StdPrg>

F1:  Jump to Presentation-Plot-Program (optional).

The current x/y data will be transfered to this program. In this

program you can plot some curves (different arrays) with automatic

window size. Here you can also plot text and boxes in the plot.

Many fonts (bitmap and vector) are available. 1 until 4 layers

are possible, so that you can plot some different plots on the

screen, printer or plotter.

F2:  Call of the window/zoom menu. Using this menu you can change the

x-axis, y-axis parameter, plot symbols, etc..

F3:  Call of the Edit-Plot-Program (optional).

The actual x/y data will be transfered to this program. The axis

information will only be overtaken if you have checked it.

This enables you to create a special plot in the edit plot

program and show only the data fitting in this window. It also

do have a possibility to buffer data and compare this data with the

actual one's. More information in that edit plot program.

F4:  Creates the last plot again.

F5:  Call of the interpolation and zoom menu. It enables you to interpolate

and smooth data and zoom the window. This function is only working on

an one curve plot.

F7:  The data will be plotted on an x/y plotter or printer. You can also save

the picture in a file as HPGL, PCL, EPS, DXF, PCX, BMP and ASCII.

Before  the printing out you will be asked for the main plotter/printer

and plot parameter. The data will be plotted with a linear regression

line if done the linear regression on screen.

F8:  Linear regression. The cursor keys will move the marking line.

Cursor right / left moves slowly, cursor up / down quick.

The enter key will define the the data for the linear regression.

The calculated data will be shown on top of the plot or on the right

side. If this function is used after data plot on plotter/printer, the

linear regression line will be drawn on the plotter and the calculated

data printed out. The linear regression is only available on special

plots, where the linear regression makes sense.

F9:  Compare energy and capture corss section of Arrhenius, tempscan, ITS and

transient with library data (optional).

A1:  (ALT-F1) This information.

A2:  List the full plot possibilities at the left screen side. You get the

same reaction if you klick with the mouse above of <StdPrg>.

If all possiblities are listed the you get by A2 the small status line

information.

A3:  Take over evaluation data as simulation data or sample parameter and

save the evaluation data in a data bank file (optional).

A4:  Mark data with a marker or plot data from the edit plot program for

comparison.

A8:  Manual linear regression. You can select the text line for regression

data and switch to manual regression. In this case you can define

the start end end point (!) of the linear regression.

A9:  Fit of Arrhenius, tempscan, ITS or transient data with simulation data

(optional).

A10: As F4 but protocols the next plot actions. This means that every curve

(array) will be saved in a buffer and all screen text will be saved in

a file. These data and text buffer then can be readed in the presentation

plot program.

A11: Jump to the screen tools for hardcopy, WINDOWS clipboard and hardcopy

and text buffer for the presentation program.

A12: Input of ASCII data from the keyboard to a special file which can be

readed in the edit plot program.

ESC, ENTER : End of the plot and jump to the menu before.

5.2  Interpolation program <InterPrg>

F2:  Call of the window/zoom menu. Using this menu you can change the x-axis,

y-axis parameter, plot symbols, etc.. This function is only available

if you call the interpolation program from the standard auto plot

program (StrPrg).

F3:  Go back to the the edit plot program. The x/y data and the changed

window will be transfered to the edit plot program.

F4:  Go back to the the edit plot program. Only the changed window will be

transfered to particular plot program.

F5:  Input of the zooming-, smoothing- and interpolation-parameters.

Using these parameter set for the actual data.

F7:  Output of the interpolated/smoothed and, if wanted, original data

on plotter/printer.

A1:  (ALT-F1) This information.

A2:  List the full plot possibilities at the left screen side.

A11: Jump to the screen tools for hardcopy, WINDOWS clipboard and hardcopy

and text buffer for the presentation program.

A12: Input of ASCII data from the keyboard to a special file which can be

readed in the edit plot program.

ESC: End of the plot and jump to the menu before.

5.3  Presentation-Plot-Program <PresPrg>

F1:  Parameter for the boarder.

F2:  Call of the window/zoom menu. Using this menu you can change the x-axis

and y-axis parameter.

F3:  Jump to the parameter menu.

F4:  Creates the last plot again.

F5:  Define a text for the plot.

F6:  Define a bitmap, box, line or symbol for the plot.

F7:  The data will be plotted on an x/y plotter or printer. You can also save

the picture in a file as HPGL, PCL, EPS, DXF, PCX, BMP and ASCII.

Before  the printing out you will be asked for the main plotter/printer

and plot parameter. The data will be plotted with a linear regression

line if done the linear regression on screen.

F8:  Linear regression.

A1:  This information.

A2:  List the full plot possibilities at the left screen side.

A3:  Input of the last selected parameters.

A4:  Mark data with a marker or plot data from the edit plot program for

comparison.

A5:  Move a text of the plot.

A6:  Move a box of the plot.

A8:  Manual regression.

A11: Jump to the screen tools (hardcopy).

A12: Input of ASCII data from the keyboard to a special file which can be

readed in the edit plot program.

ESC, ENTER : End of the plot and jump to the menu before.

5.4a  Edit-Plot-Program <EditPrg>, main mode

F1:  Use buffered data

F1: buffer   I (standard buffer)

F2: buffer  II (special buffer)

F3: buffer III (source data)

F4: buffer  IV (old edit data)

F5: buffer   V (last data)

F2:  Call of the window/zoom menu. Using this menu you can change the x-axis

and y-axis parameter.

F3:  Input of x/y axis. Create new plot with the actual data.

F4:  Show plot again.

F5:  Jump to the interpolation program for zooming and interpolation and

smoothing of the current data.

F6:  Show difference between actual data (global plot array) and data from

buffer or interpolated / smoothed data of the actual data.

x/y values can also transfered to the global plot array.

F1: Difference between current data and buffer   I (standard buffer)

F2: Difference between current data and buffer  II (special buffer)

F3: Difference between current data and buffer III (source data)

F4: Difference between current data and buffer  IV (old edit data)

F5: Interpolation of the current data.

F7:  Print plot on plotter, printer or file.

F8:  Linear regression.

F9:  Switch from the main mode to the edit mode. Here you can edit, mark,

change, delete or add data. Use info key in that mode for more help.

A1:  This information.

A2:  List the full plot possibilities at the left screen side.

A3:  Take over evaluation data as simulation data or sample parameter and

save the evaluation data in a data bank file (optional).

A4:  Mark data with a marker or plot data from the edit plot program for

comparison.

A8:  Manual regression.

A10: As F4 but protocols the next plot actions.

A11: Jump to the screen tools for hardcopy, WINDOWS clipboard and hardcopy

and text buffer for the presentation program.

A12: Input of ASCII data from the keyboard to a special file.

ESC, ENTER: End of this program, go to the previous menu.

5.4b  Edit-Plot-Program <EditPrg>, full edit mode

F1:  Delete the data at mark line (use cursor keys moving the marker).

Deleted data will stay on screen till the plot is created new (F4).

F2:  Deleted data will be valid again if the new plot function F4 has not be

used before.

F3:  Input of x/y axis. Create new plot with the actual data.

F4:  Create new plot of the actual data. As deleted data marked data will

be taken out, added data will be taken into the plot array.

F5:  Mark x/y-field. First-x value will be the data the marker is showing.

Move marker to the last x-value of the field and hit enter.

x/y-field is defined including start and end value.

If the last x-value is < than the first one the field is not defined.

Following functions are available. (for the field):

F1: Delete the field.

F5: Add a constant value to the x-values in the field.

F6: Multiply the x-values in the field with a constant value.

F7: Add a constant value to the y values in the field.

F8: Multiply the y values in the field with a constant value.

Any key : leave every data unchangend

F6:  The marked data can be set on a new position using the editor.

(input of x/y positon). The marker will move to the new positon.

The data point will stay on screen till using F2 or F4.

F7:  The marked data point can be changed by moving the marker (use cursor

keys) to the new position and hit 'enter'.The new position will be

marked with a cross.

F8:  Add a new data point using the marker (similar to F7).

F9:  Go back to main mode of the plot.

A1:  This information.

A2:  List the full plot possibilities at the left screen side.

A3:  New plot, but remains in this edit mode.

A4:  Mark data.

A10: As F4 but protocols the next plot actions.

A11: Jump to the screen tools for hardcopy, WINDOWS clipboard and hardcopy

and text buffer for the presentation program.

A12: Input of ASCII data from the keyboard to a special file.

ESC: End of the plot, go back to the last menu.

5.4c  Edit plot program <EditPrg>, part edit mode

F3:  Input of x/y axis. Create new plot with the actual data.

F4:  Create new plot of the actual data. As deleted data marked data will

be taken out, added data will be taken into the plot array.

F6:  The marked data can be set on a new position using the editor.

(input of x/y positon). The marker will move to the new positon.

The data point will stay on screen till using F2 or F4.

F7:  The marked data point can be changed by moving the marker (use cursor

keys) to the new position and hit 'enter'.The new position will be

marked with a cross.

F9:  Go back to main mode of the plot.

A1:  This information.

A2:  List the full plot possibilities at the left screen side.

A3:  New plot, but remains in this edit mode.

A4:  Mark data.

A10: As F4 but protocols the next plot actions.

A11: Jump to the screen tools for hardcopy, WINDOWS clipboard and hardcopy

and text buffer for the presentation program.

A12: Input of ASCII data from the keyboard to a special file.

ESC: End of the plot, go back to the last menu.

5.4d  Edit plot program <EditPrg>, part edit mode 2

F1:  Delete the data at mark line. (use cursor keys moving the marker.)

Deleted data will stay on screen till the plot is created new (F4).

F2:  Change of x/y axis. Creates new plot using the new axis's.

Added x/y data will be shown as well as deleted one's.

F3:  Deleted data will be valid again if the new plot function F4 has not be

used before.

F4:  Create new plot of the actual data. As deleted data marked data will

be taken out, added data will be taken into the plot array.

F5:  Delete a x/y field. The field limits will be set by the marker.

First value will be the actual marker position, last value will be set

by the marker (using the cursor keys) and hitting 'Enter'.

First and last values willalso be deleted. Field definition is only

valid if first value < last value.

F6:  The marked data can be set on a new position using the editor.

(input of x/y positon). The marker will move to the new positon.

The data point will stay on screen till using F2 or F4.

F7:  The marked data point can be changed by moving the marker (use cursor

keys) to the new position and hit 'enter'.The new position will be

marked with a cross.

A10: As F4 but protocols the next plot actions.

A11: Jump to the screen tools for hardcopy, WINDOWS clipboard and hardcopy

and text buffer for the presentation program.

A12: Input of ASCII data from the keyboard to a special file.

ESC: Go back to last menu.

5.5  Application plot program <ApplPrg>

F1:  Jump to the presentation program (not always possible).

F3:  Jump to the edit plot program (not always possible).

F4:  Creates the last plot again.

F5:  Save data (optional).

F7:  The data will be plotted on an x/y plotter or printer. You can also save

the picture in a file as HPGL, PCL, EPS, DXF, PCX, BMP and ASCII.

Before  the printing out you will be asked for the main plotter/printer

and plot parameter. The data will be plotted with a linear regression

line if done the linear regression on screen.

F8:  Linear regression. The cursor keys will move the marking line.

Cursor right / left moves slowly, cursor up / down quick.

The enter key will define the the data for the linear regression.

The calculated data will be shown on top of the plot or on the right

side. The linear regression is only available on special plots, where

the linear regression makes sense.

A1:  (ALT-F1) This information.

A2:  List the full plot possibilities at the left screen side. You get the

same reaction if you klick with the mouse above of <StdPrg>.

If all possiblities are listed the you get by A2 the small status line

information.

A3:  Take over evaluation data as simulation data or sample parameter and

save the evaluation data in a data bank file (optional).

A10: As F4 but protocols the next plot actions. This means that every curve

(array) will be saved in a buffer and all screen text will be saved in

a file. These data and text buffer then can be readed in the presentation

plot program.

A11: Jump to the screen tools for hardcopy, WINDOWS clipboard and hardcopy

and text buffer for the presentation program.

A12: Input of ASCII data from the keyboard to a special file which can be

readed in the edit plot program.

ESC, ENTER : End of the plot and jump to the menu before.

5.6  Parameter plot

F1:  Jump to Presentation-Plot-Program (optional). All x/y data will be

transfered to this program.

F2:  Call of the window menu. Using this menu you can change the x-axis

and y-axis parameter.

F3:  Make a plot with the data of a vertical line in the current plot.

F4:  Creates the last plot again.

F7:  The data will be plotted on an x/y plotter or printer. You can also save

the picture in a file as HPGL, PCL, EPS, DXF, PCX, BMP and ASCII.

Before  the printing out you will be asked for the main plotter/printer

and plot parameter.

F9:  Compare energy and capture corss section of Arrhenius, tempscan, ITS and

transient with library data (optional).

A1:  (ALT-F1) This information.

A2:  List the full plot possibilities at the left screen side.

A3:  Set a new auto window and plots again.

A4:  Mark data with a marker or plot data from the edit plot program for

comparison.

A9:  Fit of Arrhenius, tempscan, ITS or transient data with simulation data

(optional).

A10: As F4 but protocols the next plot actions. This means that every curve

(array) will be saved in a buffer and all screen text will be saved in

a file. These data and text buffer then can be readed in the presentation

plot program.

A11: Jump to the screen tools for hardcopy, WINDOWS clipboard and hardcopy

and text buffer for the presentation program.

A12: Input of ASCII data from the keyboard to a special file which can be

readed in the edit plot program.

ESC, ENTER : End of the plot and jump to the menu before.

5.7  3-dimensional plot

F2:  Call of the window menu. Using this menu you can change the x-axis

and y-axis parameter.

F4:  Creates the last plot again.

F7:  The data will be plotted on an x/y plotter or printer. You can also save

the picture in a file as HPGL, PCL, EPS, DXF, PCX, BMP and ASCII.

Before  the printing out you will be asked for the main plotter/printer

and plot parameter.

A1:  (ALT-F1) This information.

A2:  List the full plot possibilities at the left screen side.

A10: As F4 but protocols the next plot actions. This means that every curve

(array) will be saved in a buffer and all screen text will be saved in

a file. These data and text buffer then can be readed in the presentation

plot program.

A11: Jump to the screen tools for hardcopy, WINDOWS clipboard and hardcopy

and text buffer for the presentation program.

A12: Input of ASCII data from the keyboard to a special file which can be

readed in the edit plot program.

ESC, ENTER : End of the plot and jump to the menu before.

5.8  Mapping plots

F2:  Call of the window menu.

F3:  Save mapping data.

F4:  Creates the last plot again.

F5:  Define a text for the plot.

F6:  Define a bitmap, box, line or symbol for the plot.

F7:  The data will be plotted on a printer or in a file.

A1:  This information.

A2:  List the full plot possibilities at the left screen side.

A5:  Move a text of the plot.

A6:  Move a box of the plot.

A11: Jump to the screen tools (hardcopy).

A12: Input of ASCII data from the keyboard to a special file which can be

readed in the edit plot program.

ESC, ENTER : End of the plot and jump to the menu before.

6.  Structure of the main plot programs

�������������������������������������������������Ŀ

�          Standard plot program <StdPrg>         �

���������������������������������������������������

�                          �                 �

�                          �                 �

�������������������Ŀ        ��������������������Ŀ      �

� Edit plot program ��������Ĵ Interpolation plot �      �

�    <EditPrg>      ��������Ĵ program <InterPrg> �      �

���������������������        ����������������������      �

�    �                     �                 �

�    �                     �                 �

�������������������������������������������������Ŀ

�       Presentation plot program <PresPrg>       �

���������������������������������������������������

�������������������������������������������������Ŀ

�        Application plot program <AppPrg>        �

���������������������������������������������������

�                                            �

�                                            �

�������������������Ŀ                                    �

� Edit plot program �                                    �

�    <EditPrg>      �                                    �

���������������������                                    �

�    �                                       �

�    �                                       �

�������������������������������������������������Ŀ

�       Presentation plot program <PresPrg>       �

���������������������������������������������������

7.  Others

a) During measurement plots the SCROLL-LOCK key freezes in the plot. That

means that the measurement goes on and only the sceen plot will be stopped.

IF this key is on (lamp) and there is no plot on the screen then you dont

get a plot during or after measurement!

b)           E : End of the measurement

c) CTRL-B      : Breaks the measurement

d) CTRL-B      : Stops the HPGL plot (plotter), but not in all situations

e) SHIFT-RIGHT : IF you list the IEEE commands and hold this key then the

program works slower so that you can see the commands

f) CTRL-SHIFT  : IF you list the IEEE commands and hold this keys then the

commands will be written to the file MEASURE.REP