******************************************************************************

*                       Error at program start                               *

******************************************************************************

Here are some tips for help if the program terminates by an error.

You should try the first 6 calls in the order below:

1) For the test of the PC configuration call the program DLSET INFO.

2) For the further test of the hardlock key call DLSET PORT.

3) To avoid IEEE error test first the simulation program DLSET SIM NOPERI.

4) To work without EMS and ramdisk start the DLTS program by DL OFF.

5) To check the point of error start the DLTS program by DL TEST.

6) If the program shall not break by an error, start by DL IGNORE.

7) To work without the cryo system start the DLTS program by DL NOCRYO.

8) If you want to go only in the IEEE command menu, start by DL IEEE.

9) For the PCI IEEE board you have to set after the port to 2b8 by pciconf.

You can combine all command line parameters of the DLTS program.

Further information is in the install manual and in the file DOC\INSTALL.DOC.

All error messages occured at program start are in the file DOC\STARTERR.REP.

The PC configuration is, after call of DLSET INFO, stored in the file

DOC\PC_CONF.REP.

******************************************************************************

*                      Problems with the hardlock key                        *

******************************************************************************

At problems with the hardlock key call DLSET INFO and, if necessary,

DLSET PORT.

With the hardlock key there are some kinds of error possible:

1) You have mismatch the hardlock key for simulation and measurement.

Some tips are:

i) The list of PC configuration detect this error. Change the hardlock

key or the installation.

2) The hardlock key and the file for the hardlock key has not the same

license number. This case is very rare.

Some tips are:

i) In this case you have normally different licence numbers at point 7

and 8 at the list of PC configuration.

ii) Load from disk the program TEST_KEY.

iii) Load form disks at other computers the program TEST_KEY, so that

you are sure that this is not a problem of the parallel port.

3) The hardlock key dont work together with a printer on the same

parallel port.

Some tips are:

i) Please switch on the printer and call again DLSET INFO.

ii) Disconnet the printer and put away the cable from the parallel

port and call DLSET INFO again.

4) The hardlock key dont work at the parallel port if the ECP/EPP mode of this

port is switched on:

Some tips are:

i) Switch off the parallel ECP/EPP and set SPP mode in the BIOS

or  ii) Set in the file AUTOEXEC.BAT: HL_SEARCH=378e,278e,3BCe

or iii) Call DLSET PORT

5) The hardlock key dont work at the parallel port and/or, if you have the

Hardlock key II, at the serial port.

Some tips are:

i) Call DLSET PORT

ii) Set in the file AUTOEXEC.BAT: HL_SEARCH=Port+PortMode

Parallel ports: 378,278,3BC

Serial   ports: 3F8,2F8,3E8,2E8

PortMode      : p = parallel

s = serial

e = parallel in ECP mode

n = NEC Japan

c = Compaq Contura docking base

i = IBM PS2

Examples      : 378,278,3BC (default without setting of HLSEARCH)

: 387e (for parallel port 378 with ECP mode)

: 3F8s (for serial port 3F8)

6) The hardlock key dont work at the parallel port because the parallel

port is not okay. This case is very rare. Only at some noname computer

with cheap IDE combi controller can occur this problem.

Some tips (after change run DLSET INFO and DLSET PORT):

i) Use if possible an other parallel port.

ii) Check, whether a printer works at this port.

iii) Check the address of the parallel port.

iv) Build in an additional parallel card for the PC.

******************************************************************************

*                       Error with the IEEE                                  *

******************************************************************************

Tips for IEEE problems:

1) Check all connection, cables. Are all IEEE devices switched on?

2) Switch all IEEE devices including the computer off, then on and try it

again.

3) Call DLSET INFO and look whether the IEEE card in the PC is installed.

4) Have you a Keithley IEEE or a compatible card with the NEC chip?

Is this card in the computer? If you haved switched on the ROM (default),

then use the card a valid address? Have you exclude in this case this

address range at the EMM driver in the file CONFIG.SYS?

5) IF you have a plotter start the simulation program DL SIM. Then plot

a picture on the plotter. If it works then the IEEE card of the computer

is okay.

6) Start the program with DL NOCRYO. If it works then the IEEE cards of the

computer and of the DLTS system are okay and your problem depends on the

temperature controller. If it dont work then the problem depends on the

computer or the DLTS system.

7) Start the program DL IGNORE. All errors should be ignored. Look at the

program start to the error messages. If you get an error only from the

cryo system the problem depends on the temperature controller. If you get

an error only from the DLTS system then the problem depends on the DLTS

system. If you get an error from the cryo system and from the DLTS system

the problem depends on the computer.

8) Start the program DL IGNORE. All errors should be ignored. Then try to

read the temperature with ALT-F5. If it works the IEEE problem depends

on the DLTS system. If it dont work the problem depends on the computer

or temperature controller.

9) Instead of (5) until (8) you can switch off for testing the device at

which the IEEE occurs. You get after the error message a question to

repeat, go on or switch off.

If you get an error only from the cryo system the problem depends on the

temperature controller. If you get an error only from the DLTS system then

the problem depends on the DLTS system. If you get an error from the cryo

system and from the DLTS system the problem depends on the computer.

10) Special problems for the PCI IEEE board:

You have to set the port, normally 2b8, after every boot by the Keithley

program PCICONF. You find this program on your Keithley disk and in the

directory \DLTS\DLTFS\INSTALL. The best is to include the call in the

AUTOEXEC.BAT file: \DLTS\DLTFS\INSTALL\PCICONF 2b8. Under Windows you can

set the port also at DLTS program start by the program/link DLTSCMD from

your desktop.

At Windows you have to initializate the plug and play IEEE board after

installing the board by the configuration file CEC488.INF. Please have

a look in your Keithley manual.

The PCI IEEE board dont run in a DOS box of Windows NT.