******************************************************************************

******************* BANK.DOC (Version 3.4.0, = 4.0 DOS) **********************

******************************************************************************

All data bank files has the standard data bank format of DBASE III with

memo fields (header signature 83H). The extensions for the data bank are

always DBF, the extensions for the memo text files are DBT. The DLTS program

dont use this file but you can use it by other data bank programs.

****************************************************************************

*              Group data bank GROUPS.DBF                                  *

****************************************************************************

I) Total header       :  353 Bytes

1) Main header       :   32 Bytes

2) Field description :  320 Bytes by 10 fields a 32 Bytes

3) Header terminator :    1 Byte

II) Record             :  384 Bytes

0) DelChar           :    1

1) Group             :   64

2) Common path       :   64

3) User path         :   64

4) Current path      :   64

5) Mode of path      :    1 - N1

6) User name         :    8

7) Reserved          :   40

8) Date              :    8 - D

9) Comment           :   60

10) Memo              :   10 - MEMO

****************************************************************************

*              ID data bank IDS.DBF                                        *

****************************************************************************

I) Total header       :  385 Bytes

1) Main header       :   32 Bytes

2) Field description :  352 Bytes by 11 fields a 32 Bytes

3) Header terminator :    1 Byte

II) Record             :  256 Bytes

0) DelChar           :    1

1) ID                :   10

2) Sub group         :   64

3) File name         :   64

4) Mode of path      :    1 - N1

5) User name         :    8

6) Mode of auto name :    1 - N1

7) Mode of name      :    1 - N1

8) Reserved          :   28

9) Date              :    8 - D

10) Comment           :   60

11) Memo              :   10 - MEMO

****************************************************************************

*              File data bank FILES.DBF                                    *

****************************************************************************

I) Total header       :  449 Bytes

1) Main header       :   32 Bytes

2) Field description :  416 Bytes by 13 fields a 32 Bytes

3) Header terminator :    1 Byte

II) Record             :  256 Bytes

0) DelChar           :    1

1) ID                :   10

2) Contact           :    2

3) File name         :   64

4) Mode of path      :    1 - N1

5) User name         :    8

6) Mode of measure   :    2 - N2

7) Type of measure   :    2 - N2

8) Numbers of files  :    3 - N3

9) Header string     :   40

10) Reserved          :   45

11) Date              :    8 - D

12) Comment           :   60

13) Memo              :   10 - MEMO

****************************************************************************

*             Evaluation data bank EVALS.DBF                               *

****************************************************************************

I) Total header       :  833 Bytes

1) Main header       :   32 Bytes

2) Field description :  800 Bytes by 25 fields a 32 Bytes

3) Header terminator :    1 Byte

II) Record             :  320 Bytes

0) DelChar           :    1

1) ID                :   10

2) Contact           :    2

3) File name         :   64

4) Mode of path      :    1 - N1

5) User name         :    8

6) Form of eval      :    2 - N2

7) Mode of eval      :    2 - N2

8) Type of eval      :    2 - N2

9) Number of eval    :    2 - N2

10) Name of eval      :   10

11) Temperature       :    6 - N6,2

12) Size              :   10

13) Correlation       :    8

14) Slope             :   10

15) Section           :   10

16) Value 1           :   10

17) Value 2           :   10

18) Value 3           :   10

19) Value 4           :   10

20) Value 5           :   10

21) Value 6           :   10

22) Reserved          :   36

23) Date              :    8 - D

24) Comment           :   60

25) Memo              :   10 - MEMO

------------------------ Mode of eval --------------------------------------

2) Line with slope and section

3) Energy from slope of Arrhenius like

4) Arrhenius with trap concentration

5) Arrhenius

6) Richardson plot

7) V % I curve, resistance

8) I % V curve, resistance

9) I % V curve, diode, n-factor

10) C % V curve

11) Zerbst

12) Nss energy

13) Capture regression

15) Time constant

----------------- Meaning of values depending of eval mode -------------------

3) Slope   : energy [eV]

4) Slope   : energy [eV]

Section : capture cross section from Arrhenius [cm^2]

Value 1 : trap concentration [cm^-3]

Value 2 : exact trap concentration [cm^-3], not always

Value 3 : used shallow concentration [cm^-3]

5) Slope   : energy [eV]

Section : capture cross section from Arrhenius [cm^2]

6) Slope   : barrier height [eV]

Section : Richardson constant

7) Slope   : resistance [Ohm]

Section : U0 [V]

8) Slope   : resistance [Ohm]

Section : I0 [uA]

9) Slope   : n-factor

Section : saturation current [uA]

Value 1 : barrier height [eV]

Value 2 : diffusion voltage [V]

10) Slope   : shallow concentration [cm^-3]

Section : diffusion voltage [V]

Value 1 : barrier height [eV]

11) Slope   : gm [cm^-3s^-1]

Section : Gb [cm^-2s^-1]

Value 1 : Life time [s]

12) Slope   : energy [eV]

Section : capture cross section [cm^2]

13) Slope   : capture time constant [s]

Value 1 : capture cross section [cm^2]

15) Slope   : time constant [s]

Section : amplitude [pF,V,uA]

Value 1 : trap concentration [cm^-3]

Value 2 : exact trap concentration [cm^-3], not always

Value 3 : used shallow concentration [cm^-3]

Value 5 : calculated energy [eV]

Value 6 : calculated capture cross section [cm^2]

The size (record position 12) means the contact area [cm^2].