******************************************************************************

********************** LIB.DOC (Version 3.4.0, = 4.0 DOS) ********************

******************************************************************************

Directory of this file:

1) Overview of files in the subdirectory LIB

2) Data bank file format of the library

3) Meaning of some special fields

******************************************************************************

*                          Files in LIB                                      *

******************************************************************************

The following library files are in the subdirectory \DLTS\LIB:

LIBRARY.DBF  : Current data bank file of the library

LIB_ORG.DBF  : Original data bank file (no own modification). On the disk

this is the file LIBRARY.DBF

LIB_OLD.DBF  : Old data bank file from the last version before updating

(not on disk)

LIBRARY.DBT  : Memo text of library. The DLTS program dont use this file

but you can use it by other data bank programs.

LIB_VERS.DAT : File with information about the current library (not on disk).

Dont change this ASCII file!

LIBTOASC.EXE : Program which transfer a library data bank file in an ASCII

or TEX file. You get the options with LIBTOASC ?. For more

information have a look in the file LIBTOASC.CON.

LIBTOASC.CON : Configuration file (ASCII) for the program LIBTOASC.

LIBTOASC.STY : Styles and text commands for the program LIBTOASC.

****************************************************************************

*               Description of library file format                         *

****************************************************************************

The library file has the standard data bank format of DBASE III with

memo fields (header signature 83H).

I) Total header       : 1057 Bytes

1) Main header       :   32 Bytes

2) Field description : 1024 Bytes by 32 fields a 32 Bytes

3) Header terminator :    1 Byte

II) Record             :  512 Bytes

0) DelChar           :    1

1) LicenseNr         :    3 - N3,0

2) IDNumber          :    4 - N4,0

3) ReferenceID       :    7

4) Name              :    8

5) Element           :    5

6) Material          :    8

7) Doping            :    1 - N1

8) Carrier           :    1 - N1,0

9) Literature        :   50

10) Date              :    8 - D

11) ModifyState       :    1 - N1,0

12) Group             :   40

13) UserName          :   27

14) Energy-Arrhenius  :    5 - N5,3

15) Energy-Sigma      :    6

16) Sigma-Arrhenius   :    9

17) Sigma-Capture     :    9

18) EffectiveMass     :    9

19) TempExponent      :    4

20) TempRange         :    7

21) TempMaximum       :    5

22) TauMaximum        :    9

23) Priority          :    1 - N1,0

24) RefStatus         :    1 - N1,0

25) Measure           :    1 - N1,0

26) Evaluation        :    1 - N1,0

27) Origin            :    1 - N1,0

28) Information       :    4 - N4,0

29) Process           :    4 - N4,0

30) Comment           :  240

31) Reserved          :   22

32) Memo              :   10 - MEMO

****************************************************************************

*           Internal fields (not in the data bank file)                    *

****************************************************************************

31) Total energy

32) Sigma-Arrhenius, corrected by effective mass

33) Full ID = IDNumber + LicenseNumber

34) Calculated maximum temperature

35) Calculated emission time constant

These fields are not in the library file. In the library file there are

32 fields but only 30 will be used in the DLTS program. The last two

fields will be readed and saved but not modified by the DLTS program

in this DOS version. Therefore the internal field numbers start at 31.

****************************************************************************

*               Meaning of some special fields                             *

****************************************************************************

Carrier     1: trap level (normal)

2: recombination center

3: p+n resp. n+p artefakt level

RefStatus   1: Main

2: Main, other values

3: Similiar values

4: Same values as 1, 2 or 3

Measure     1: unknown

2: DLTS

3: DLTFS

4: DLTFS with own measure-cryo system

Evaluation  0: unknown

1: direct DLTFS, variable period width, Arrhenius

2: direct DLTFS, constant period width, Arrhenius

3: tempscan maxima, Arrhenius

4: isothermal, Arrhenius

5: sigma_A from energy, T_max and tau_max

6: direct DLTFS, only one temperature point

7: tempscan maximum, only one temperature point

8: isothermal, only one temperature point

Origin      0: unknown

1: metall

2: metall, complex with intrinsic

3: metall, complex with extrinsic-not-metall

4: metall, complex with extrinsic-metall

5: metall, complex with shallow doping

6: extrinsic, not metall

7: intrinsic

8: extrinsinc and intrinsic

9: extrinsic/intrinsic with shallow doping

Process

Bit 0   (1): not thermical stable

Bit 1   (2): not used

Bit 2   (4): with heat/annealing

Bit 3   (8): RTA

Bit 4  (16): not used

Bit 5  (32): meta stable, with UR

Bit 6  (64): irradiated

Bit 7 (128): H-passivation

Bit 8 (256): stress

Bit 9 (512): native

****************************************************************************

*              Meaning of special fields in the comment                    *

****************************************************************************

Special fields (perhaps for searching) exist in the comment. The key words

are in capital letters and end with a "=". The normal comment follows after

"//".

1) NAME       : other name

2) ID         : identification of trap

3) CHAR       : a (aceptor), d (donor), aa, dd

4) PLACE      : i (interstitial), s (substitional)

5) IMPORT     : important if related levels (1:very, 5:not)

6) NT/NS      : ratio of concentration

7) NS         : shallow concentration

8) SAMPLE     : Schottky, p+n, ...

9) MAT        : material as B-FZ, CZ, LEC, ...

10) MEAS       : measure technique

11) DIFTEMP    : temperature for diffusion

12) ANNEAL     : annealing

12) ANNEAL_IN  : in at annealing temperature

14) ANNEAL_OUT : out at annealing temperature

15) SIGMA_A    : information for capture cross section by Arrhenius plot,

e.g. "calc." means that this value was not in the paper

but calculated with energy, temperature and time constant

16) SIGMA_C    : information for capture cross section by capture measurement

17) TAU        : (second) emission time constant

18) TEMP       : (second) temperature

19) ENERGY_TX  : temperature exponent for energy calculation, only

interesting if sigma was calculated

Example:

CHAR=a, SAMPLE=Schottky // This is an example