3.4.0

05.06.1997

Directory

1. General data description and overview of data files

2. Converting data by the program DLASC

3. Description of the data header

4. Description of the PTV-block

5. Description of the Arrhenius files

6. Description of the equilibrium files

7. Description of edit plot files

8. Description of header string and extensions

*****************************************************************************

*                    1. General data description                            *

*****************************************************************************

The C/V-I/V data (*.K*), the transient data (*.Y*), the isothermal data (*.P*),

the tempscan data (*.T*,*.V*), the Maximum-Arrhenius data (*.A*) and the

equlilibrium data (*.Q*) are normally binary data. The file start with

a header of 512 bytes. After this begins the measurement data block.

At C/V-I/V data this block starts with the first voltage [V], then with the

first capacitance [pF] or the first current [uA]. Then follows the second

voltage and so on. All data are 4 Byte REAL data.

The transient data block is the measured transient array y [pF,V,uA]. It

starts with y[0], y[1],... and goes to y[N], where N is the number of sampling

points (power of two). All data are 4 Byte REAL data.

The isothermal data and the tempscan data has a similiar format. The tempscan

data itself depend on the selected file format. But all formats have the same

structure and based on a PTV-data block. This data block are repeated at

varying the temperature, the voltage, pulse width or period width. The block

has a langth of 144 Bytes and contains 35 REAL-data and 2 INTEGER-data.

The plot files (*.X*), made in the edit plot program, are ASCII-files.

*****************************************************************************

*               Dimensions and ASCII files                                  *

*****************************************************************************

In all files the dimension values will stored as:

Temperature   : [K]

Voltage       : [V]

Capacitance   : [pF]

Current       : [uA]

Time          : [s]

Length        : [cm]

Area          : [cm^2]

Volumen       : [cm^3]

Concentration : [cm^-3]

Energy        : [eV]

Frequency     : [Hz]

You can get from the binary data ASCII data in several ways:

1) All binary files you can transform with the external DLASC program in ASCII

files (see below). You get also an information by DLASC ?.

2) You can store data in the edit plot program as plot files (see below) with

a small header, menu E.5.2.

3) You can store data in the edit plot program as pure ASCII column data

without a header, menu E.5.3. The x and y values are in the same line,

separated by a delimiter, normally a blank. You can change the delimiter

here at the inputs. You can also also choice whether you want to numerate

the lines.

4) In the presentation plot program you can as in (3) save ASCII data. In the

menu P.5.2 an x/y-array and in menu P.5.3/4 a ASCII table.

5) In the file menu A.2.4 you can store the the x-y-array of the last screen

plot as a pure ASCII file as in (3).

6) At the plots you can can store the plot by F7 in a file. Here you can

select ASCII file , as (3), if there exist only one curve.

7) At listing data you can can store the data in an ASCII, as (3), file by F7.

*****************************************************************************

*                       Overview of data files                              *

*****************************************************************************

1) Plot file (*.X*)          : plot header (20 ASCII lines),

x[1],y[1], ...., x[N],y[N] (ASCII)

2) C/V,I/V-file (*.K*)       : data header (512 Bytes),

U[1],C[1], ..., U[N],C[N] (REAL)

3) Transient file (*.Y*)     : data header (512 Bytes),

y[0], ..., y[N] (REAL)

4) Isothermal file (*.P*)    : data header (512 Bytes),

PTV-block[1], ..., PTV-block[N] (REAL)

5) Tempscan file (*.T*,*.V*) : data header (512 Bytes),

PTV-block[1], ..., PTV-block[N] (REAL)

6) Arrhenius file (*.A*)     : data header (512 Bytes),

Arrh-block[1], ..., Arrh-block[N]

7) Equilibrium file (*.Q*)   : data header (512 Bytes),

T[1],CI[1,1-4], ..., T[N],CI[N,1-4] (REAL)

8) Presentation file (*.F*)  : Binary structed file with text and data.

9) Figure/bitmap file (*.B*) : Different structures, look in WORK\FIGURES

for examples

*****************************************************************************

*                   2. Converting data by DLASC                             *

*****************************************************************************

All binary files you can transform with the DLASC program in ASCII files.

Set behind DLASC the filename, wildcards are also possible. With DLASC *.*

you can transform a complete subdirectory. Files, which are ASCII-files,

will be automatically recognized. The file with the binary data will be

overwritten. You can retransform the ASCII data in binary data by

DLASC /CR.

Normally (see options) each ASCII data will be written in a new line, that

means that every value will be separated from the other by cr-lf.

With the option C you can convert the binary data in a non DLTFS-format (table

data). In this case you cannot retransform such ASCII-tabular-data to binary

DLTFS-data because you loose the header. With the option C you get a standard

ASCII file, which many programs can read, for example DBASE and EXCEL.

The following options for DLASC exist:

Options: C --> Write the ASCII data for each block in one line and every data

here in an own column. The data in one block will be separated

by a delimiter, normally a blank. No header!

CBx --> As 'C', but use only block x.

C1  --> Write one ASCII value in one line. With header!

C2  --> As 'C1', but write all values in one line.

C3  --> As 'C1', but write blocks value in one line.

CR  --> Retransform ASCII data to binary data.

Dx  --> Define the delimiter for the columns to CHR(x), 44 for comma.

Ex  --> Select the numbers x of exponentials behind the point.

LX  --> Last line terminator (0:no, 1:cr+lf, 2:cr+lf+eof)

Nx  --> Numerate line (0:no, 1:yes, 2:starts new at block).

Default:       C1, D32(blanc), E4, L1, N0

*****************************************************************************

*               3. Description of the data header                           *

*****************************************************************************

Line, binary position and data type:

1-  1-S40: Header-String

2- 41-  I: Start index of array

3- 43-  I: End index of array

4- 45-S60: Comment                    ----------

5-105-S10: IDName

6-115- S2: Contact

7-117- S8: MaterialName

8-125-  U: Doping (n=1,p=2)

9-127-  U: Mode of sample

10-129-  R: Area of contact [cm^2]

11-133-  R: Thickness [cm]

12-137-  R: Shallow concentration [cm^-3]

13-141-  R: Oxid capacitance [pF]

14-145-S20: ModeString

15-165-  R: Activation energy [eV]     ----------

16-169-  R: Capture cross section (Arrhenius) [cm^2]

17-173-  R: Capture cross section (capture) [cm^2]

18-177-  R: Trap concentration [cm^-3]

19-181-  R: Correlation

20-185-  R: Diffusion or flatband voltage [V]

21-189-  R: OxidThick?

22-193-  R: free

23-197-  R: free

24-201-  U: Process

25-203-  I: free

26-205- S2: DopingMaterial             ----------

27-207-S10: Date

28-217- S8: Name of user

29-225- S8: IDGroup

30-233-S20: ReferenceString

31-253-S12: ReferenceName

32-265-S24: DummyString

33-289-S24: HeaderString

34-313-S20: MeasureString

35-333-S20: ParameterString

36-353   I: free                       ----------

37-355   I: free

38-357   I: free

39-359   I: free

40-361   R: free

41-363-  R: Amplification of transient signal

42-367-  R: Voltage, at which compensation takes place [V]

43-373-  R:

44-377-  R: Width of injection pulse [s]

45-381-  R: Frequency of analog anti-aliasing filter [Hz]

46-385-  R:

47-389-  R: Amplification of slow CC-regulation

48-393-  R: Amplification of fast CC-regulation

49-395-  U: Numbers of transients for average

50-397-  U:

51-399-  U: Number of first pulse

52-403-  R: Waiting time between pulses [s]

53-407-  R: Value for injection pulse [V]

54-411-  R: Temperature [K]

55-415-  R: Reverse bias [V]

56-419-  R: Pulse voltage [V]

57-423-  R: Pulse width [s]

58-427-  R: Capacitance at reverse bias [pF]

59-431-  R: Capacitance at pulse voltage [pF]

60-435-  R: Leakage current at reverse bias [uA]

61-439-  R: Temperature 2 or after measurement [K]

62-443-  R: Phase delay time of analog filter and bridge [s]

63-445-  I: Numbers of transient points

64-447-  I: Numbers of sampling intervalls between pulse and measurement

65-451-  R: Sampling interval [s]

66-453-  R: Factor Digits --> pF,V,uA

67-457-  I: Flag for index and error

68-461-  R: Factor Digits --> pF,V,uA for range 1 and amplification 1

69-465-  R: Data format dependig value x1

70-469-  R: Data format dependig value x2

71-473-  R: Data format dependig value x3

72-477-  R: Data format dependig value x4

73-481-  R:

74-485-  R:

75-489-  R: Delta voltage for slow CC-regulation [V]

76-493-  R: Drain source (auxiliary) voltage [V]

77-497-  R: Wave length

78-501-  R: Optical intensity

79-505-  R: Optical intensity factor

80-509-  R:

******************************************************************************

The header has a length of 512 bytes in the normal binary mode and a length

of 80 lines after restoring to ASCII-format with the program DLASC.

Data type:

I: 2 Byte INTEGER

U: 2 Byte unsigned INTEGER

R: 4 Byte REAL

SX: String with X Bytes of CHAR

---------------------- Special strings --------------------------------------

ModeString (line 14):

1+2) Comment

3) DLTS-Mode (1:C, 2:U, 3:I)

5) Evaluation mode

7) Area mode (0:F+Cox, 1:d+Cox, 2:F+dox, 3:d+dox, 4:dox+Cox)

with F=area, d=diameter, dox=oxid thickness, Cox=oxid capacitance

8) Carrier (0:maj, 1:maj+recombi, 2:maj+min, 3:maj+min at light, 4:min)

9) Pulse type (0:electric, 1:optic, 2:electric+optic)

10) Set pulse type (0-2:as pulse type, 3:auto, 4:auto + light on)

MeasureString (line 34):

1) Temperature (0:no, 1:mid, 2:mid/before, 3:two,cryo=main, 4:two,sec=main)

2-4) Number of cryo system

5) Invert transient

6) BridgeOff (0:no, 1:CR old, 2:input, 3:sim)

7) Sample manipulator mode

8) Sample variation mode

9) Sample manipulator numbers

10) Maximum sample manipulator numbers

12) Type of temperature ramp

13) Type of transient recorder

14) Type of amplifier

Parameter-String (line 35):

1) Type of bridge (1:Boonton 15mV, 2:100mV)

2) Ordinate

3) DLTS-Mode (1:C, 2:U, 3:I)

4) Transient format

5) Transient DC correction

6) Slow CC-regulation

7) Fast CC-regulation

8) Mode for pulse capacitance

14) Pulse mode (0:no, 1:normal, 2:double, 3:fast, 4:double-fast,

5:optical, 6:elec.+optical, 7:optical+elec.,

8:elec.-fast+optical, 9:optical+elec.-fast)

15) Measure start (0:after pulse, 1:at pulse, auto. pulse width, 2:at pulse)

16) Auxiliary/FET mode (0:no, 1:FET, 2:aux)

17) Variable optical laser on (N/Y)

18) Intensity mode for variable laser

Flag for index and error (line 67): The first 3 Bits (Bit0 until Bit2) marks

an error. The meaning is (not bitwise):

0: no error

1: system error

2: ADC overflow

4: compensation error

The other bits marks bitwise some other characteristics:

Bit4: error at slow CC-DLTS regulation

Bit8: additional capacitance amplifier is on

Bit11-Bit13: capacitance range

Data format depending value at transient data:

x1: First transient value [pF,V,uA]

x2: Last transient value [pF,V,uA]

x3: DC-value before measurement [pF,V,uA]

x4: DC-value after measurement [pF,V,uA]

Data format depending value at tempscan data:

x1: Minimum (start) temperature, if temperature increased at measurement

x1: Maximum (start) temperature, if temperature decreased at measurement

x2: Maximum (end)   temperature, if temperature increased at measurement

x2: Minimum (end)   temperature, if temperature decreased at measurement

x4: Measure cycle time [s] (also for QR, QF and QP files)

Data format depending value at C/V,I/V and isothermal data:

x1: Start value for x-axis

x2: End-value for x-axis

x3: Step-value for x-axis

*****************************************************************************

*                    4. Description of the PTV-block                        *

*****************************************************************************

Line and binary position in the PTV-block:

1-  1: Temperature (average between before and after measurement) [K]

2-  5: Reverse bias [V]

3-  9: Pulse voltage [V]

4- 13: Pulse width [s]

5- 17: Capacitance at reverse bias [pF]

6- 21: Capacitance at pulse voltage [pF]

7- 25: Leakage current at reverse bias [uA]

8- 29: Temperature 2 or after measurement [K]

9- 33: Phase delay time of analog filter and bridge [s]

10- 37: X1 or numbers of transient points

11- 41: X2 or numbers of sampling intervals between pulse and measurement

12- 45: X3 or sampling interval

13- 49: First transient value

14- 53: Last transient value

15- 57: DC-value before measurement

16- 61: DC-value after measurement

17- 65: 0. cosine-coefficient

18- 69: Rectangular correlator signal

19- 73: a1, 1. cosine coefficient

20- 77: b1, 1. sine coefficient

21- 81: a2, 2. cosine coefficient

22- 85: b2, 2. sine coefficient

23- 89: a1L = a1 with Tw'=Tw/4

24- 93: b1L = b1 with Tw'=Tw/4

25- 97: X4

26-101: X5

27-105: a1H = a1 with Tw'=Tw/2, t0'=t0+Tw/2

28-109: b1H = b1 with Tw'=Tw/2, t0'=t0+Tw/2

29-113: a1M = a1 with Tw'=Tw/2, t0'=t0+Tw/16

30-117: b1M = b1 with Tw'=Tw/2, t0'=t0+Tw/16

31-121: DRL = Double-rectangular with Tw/4 or Dlts-signal

32-125: DR  = Double-rectangular with Tw

33-129:

34-133:

35-137: Factor Digits --> pF,V,uA

36-141: Number relative (INTEGER)

37-143: Flag for index and error (INTEGER), see header line 67

******************************************************************************

The last two values are 2-Byte INTEGER, the others are 4-Byte REAL values.

The values in line 13 until 32 are in pF, V or uA, depending on the DLTS mode.

IF the period width varyies (*.V*,*.PW*) is the second possibility valid at

position 10, 11, 12 and 32 valid. In the other case, that means constant

period width, the X1, X2, X3, X4 and X5 values in position 10, 11, 12, 25 and

26 depends on the selected format for the additional weighting signals.

All isothermal files with constand period, these are all files with the first

extension P except the *.PW*-files, have the weighting mode 2. This mode has

the name 3 Dlts, the name of mode 3 is 6 Dlts and the name of mode 4 Fourier.

In the followig table a "D" marks a DLTS-signal.

Mode      2          3           4       8

X1  -->  D1   D1=y1-y2   b1(Tw/32)   User5

X2  -->  D2   D4=y4-y5   b1(Tw/16)   User3

X3  -->  D3   D6=y7-y6   b1(Tw/8)    User1

X4  -->  a4   D5=y6-y5   a1(Tw/8)    User2

X5  -->  b4   D3=y3-y4   a1(Tw/16)   User4

In the case of weighting mode 3 there are 6 DLTS-signals, where 5 are stored in

the file. The sixth you can calculate with these five and the first and last

transient value: D2=y1-y7-D6-D5-D4-D3-D1. Here and in the table above y1 is the

first transient value, y2 the second and y7 the last. So you can reconstruct

7 transient points. The times corresponding to the transient values are:

t1=z0*td, t2=2*td, t3=4*td, t4=16*td, t5=32*td, t6=64*td, t7=(z0+N)*td

z0 is here the numbers of sampling intervals between pulse and measurement

(line 62 in the header), N the numbers of points and td the sampling interval.

This mode exist only for N=256 with z0=4 and for N=512 with z0=8.

You get additional coefficients by combining two coeffcients:

a1(Tw/2)=2*a2-a1H, b1(Tw/2)=2*b2-b1H, a2N=a2-a1H, b2N=b2-b1H

For the weighting mode 8 (user signals) there is a additionaly information

about the time constant and the amplitude factor necessary. This information

is saved in the PTV-block 0. Normally the main data start with the PTV-block

1 and goes to the PTV-block N, where N is the number of temerature points.

With the weighting function 8 the data after the header start with the

PTV-block 0, where the additionally information are stored. Then follows the

PTV-block 1 with the first main data. In case of weighting function 8 the

position of the PTV-block 0 has the following meaning:

10) Factor of amplitude for user signal 5 (X1)

11) Factor of amplitude for user signal 3 (X2)

12) Factor of amplitude for user signal 1 (X3)

25) Factor of amplitude for user signal 2 (X4)

26) Factor of amplitude for user signal 4 (X5)

28) Normalized time constant for user signal 5 (b1H)

24) Normalized time constant for user signal 3 (b1L)

22) Normalized time constant for user signal 1 (b2)

21) Normalized time constant for user signal 2 (a2)

23) Normalized time constant for user signal 4 (a1L)

The factor of amplitude is the amplitude divided by the height of the maximum,

for DLTS-signals with t2=2t1 is this value 4. The normalized time constant

is the emission time constant at the maximum divided by the period width.

The emission time constant for DLTS-signals is (t2-t1)/ln(t2/t1).

*****************************************************************************

*             5. Maximums-Arrhenius-file (*.A*)                             *

*****************************************************************************

An Arrhenius-file can build from the transient capture measurement (*.AC*),

from the transient emission measurement (*.AE*), from pulse width evaluation

(*.AF*), from the period width (ITS) maximum evaluation (*.AW*), from the

tempscan maximum evaluation (*.AT*) and from the Zerbst evaluation (*.AZ*).

All files have the same structure. They consist of the standard header and

a multiple of Arrhenius-blocks. The Arrhenius-blocks for the different levels

and for the different period widths (tempscan maximum evaluation). The maximum

number of levels is 8. For each of this you can have maximal 3 Arrhenius-blocks

corresponding to various period widths.

An Arrhenius-block contains a block-header and the block-datas:

I) Block header =  80 Byte

II) Block datas  = 640 Byte

1) Record 1  =  32 Byte (temperature point 1, coefficient

...............................................

20) Record 20 =  32 Byte (temperature point 20, coefficient 20)

The block datas are a multiple of 20 follwing records:

1-R) Temperature [K]

2-R) Capacitance at reverse bias [pF]

3-R) Capacitance at pulse voltage [pF]

4-R) Factor Digits --> pF,V,uA

5-R) Shallow concentration [cm^-3]

6-R) Amplitude [pF,V,uA]

7-R) Time constant [s]

8-I) Max-class-flag (not the same meaning at direct evaluation) (INTEGER)

9-I) Additional flag

At the tempscan maximum evaluation (*.AT*-file ) there is the following

structure:

level 1, block (file) 1: Arrhenius block

"   1,   "      "   2: Arrhenius block

"   1,   "      "   3: Arrhenius block

......................

"   8,   "      "   3: Arrhenius block

The record of the Arrhenius-block origins from the different coefficients and

Dlts-signals, where the origin of some Arrhenius-blocks are depend on the

weighting mode. The order of "coeffcient" (Arrhenius-block) is for all

weighting modes (in brackets the corresponding position in the PTV-block):

1) b1H [28], class=69

2) b1 [20], class=69

3) b1M [30], class=68

4) b1(Tw/2)=2*b2-b1H [-], class=69

5) a1 [19], class=67

6) a1(Tw/2)=2*a2-a1H [-], class=67

7) a1(Tw/4) [23], class=68

16) a1M [29], class=65

17) DR  [32], class=66

18) DRL [31], class=65

For weighting mode 2 is valid:

13) D3 [12], class=60

14) D2 [11], class=60

15) D1 [10], class=61

For weighting mode 3 is valid:

10) D6 [12], class=60

11) D5 [25] , class=60

12) D4 [11], class=60

13) D3 [26], class=60

14) D2=y1-y7-D6-D5-D4-D3-D1 [-], class=61

15) D1 [10], class=61

For weighting mode 4 is valid:

8) b1(Tw/16) [11], class=69

9) b1(Tw/32) [10], class=69

10) a1(Tw/16) [26], class=68

11) a1H [27], class=65

12) b2N=b2-b1H [-], class=66

13) b1(Tw/4) [24], class=64

14) a2N=a2-a1H [-], class=66

15) b1(Tw/8) [12], class=64

For weighting mode 8 is valid:

11) user signal 1 [12], class=62

12) user signal 2 [25], class=62

13) user signal 3 [11], class=62

14) user signal 4 [26], class=62

15) user signal 5 [10], class=62

The max-class-flag has not the same meaning at direct eveluation but similiar

values because compatibility. This flag marks from which coefficient or Dlts-

signal the emission time constant was evaluated:

0: no maximum or no maximum evaluated for this point

60: Dlts-signal

61: Dlts-signal, important for Arrhenius-plot

62: user correlator signal

64: type of b1-coefficient

65: good energy resolution, part

66: good energy resolution

67: good signal noise ratio

68: all

69: all and type of b1

At the period width (ITS) maximum evaluation (*.AW*) there is the following

structure:

level 1, temperature point  1-20: Arrhenius block

level 1, temperature point 21-40: Arrhenius block

level 1, temperature point 41-60: Arrhenius block

........................................................

"   8, temperature point 41-60: Arrhenius block

The max-class-flag marks the origin-coeffcient of the maxima data:

0: no maximum or no maximum evaluated for this point

51: b1

52: a1

53: b1L

54: a1L

55: b1M

56: a1M

57: b1H

58: a1H

59: b2N

60: a2N

61: DR

62: Dlts-signal

63: ITS-signal

64: ITS-signal with C^2-correction

At the Zerbst temperature evaluation (*.AZ*) there is the following

structure:

temperature point  1-20: Arrhenius block

temperature point 21-40: Arrhenius block

temperature point 41-60: Arrhenius block

The position in the Arrhenius record means here:

1) Temperature [K]

2) Capacitance at reverse bias [pF]

3) Generation rate gm (from slope) [cm^-3s^-1]

4) Generation rate Gb (from interception) [cm^-2s^-1]

5) Shallow concentration [cm^-3]

6) Amplitude of transient [pF,V,uA]

7) Hold time [s]

8) Flag (0:no valid data, 50-53:valid data) (INTEGER)

9) Additional flag

*****************************************************************************

*             6. Equilibrium-file (*.Q*)                                    *

*****************************************************************************

The eqilibrium file contains of the standard data header and a multiple of

the equilibrium data block.

Line and binary position in the PTV-block:

1- 1: Temperature (average between before and after measurement) [K]

2- 5: Temperature 2 or after measurement [K]

3- 9: Value 3 (REAL)

4-13: Value 4 (REAL)

5-17: Value 5 (REAL)

6-17: Value 6 (REAL)

The meaning of the values 3 until 6 depends on the equilibrium mode.

Mode 1(R), 2(F), 3(P):

Value 3 : capacitance [pF]

Value 4 : current [uA]

Value 5 : 0

Value 6 : 0

Mode 11(C):

Value 3 : capacitance at UR [pF]

Value 4 : capacitance at UP [pF]

Value 5 : diffusion voltage [V]

Value 6 : shallow concentration [cm^-3]

Mode 13(I):

Value 3 : current at UR [uA]

Value 4 : current at UP [uA]

Value 5 : saturation current [uA]

Value 6 : n-factor

*****************************************************************************

*              7. File of edit plot program (*.X*)                          *

*****************************************************************************

Line:      1-S40: HeaderString

2-  I: Start index of the arrays (IS)

3-  I: End index of the arrays (IE)

4-S60: Comment

5-S10: ID

6- S2: Contact

7- S8: Material

8-  U: Doping type

9-  U: Sample type

10-  R: Area [cm^2]

11-  R: Thickness [cm]

12-  R: Shallow concentration [cm^-3]

13-  R: Oxid capacitance [pF]

14-S20: ModeString

15-  R: Temperature [K]

16-  R: Capacitance at reverse bias [pF]

17-  I: Reserved

18-  R: Reserved

19-  R: Reserved

20-  R: Reserved

After this 20 header lines follow the data. The x and y values are either

in one line and seperated by a blanc:

Line:            21: x[IS] y[IS]     = 1. value of x- and y-array

22: x[IS+1] y[IS+1] = 2. value of x- and y-array

................................................

IE-IS+21: x[IE] y[IE]     = last value of x- and y-array

The other possibility is that the x and y values are in seperated lines:

Line:            21: x[IS]           = 1. value of the x-array

22: y[IS]           = 1. value of the y-array

23: x[IS+1]         = 2. value of the x-array

24: y[IS+1]         = 2. value of the y-array

................................................

2*(IE-IS)+21: x[IE]           = last value of the x-array

2*(IE-IS)+22: y[IE]           = last value of the y-array

All plots (curves) in the DLTFS-program you can store in this plot file

format. Go during a plot with the F3-key to the edit plot program and

store there with F5 the data. The plot file is an ASCII-file.

The lines 5 until 20 are normally not important.

The header is a string of 40 characters:

7- 8: data version at make time

9-11: data version at save time

13: main type of format (always 1)

18: store type

19: mode of regression

20: submode of regression

Store type:

1: The x- and y-data are in separated lines.

2: The x- and y- data of one point are in the same line, but in different

columns, separated by a blank or other delimiter.

3: Like 2, but additional starts every data line with the number of point.

Mode of regression:

0,1,2: no         --> slope, section of axis

3: Energy     --> dE

4: Arrhenius  --> dE, sigma, NT (NT not availble after reading data)

5: Arrhenius  --> dE, sigma

6: Richardson --> phib

7: V/I-resi   --> resistance

8) I/V-resi   --> resistance

9: I/V        --> n-factor, phib

10: C/V        --> Ns, UD

11: Zerbst     --> gm, Gb

12: Nss        --> sigma

13: Capture    --> tauC, sigmaC

The indexes of the plot-arrays can be in the range form 0 until 1024, IE>=IS.

The normal start index is 1. For special functions, for example Fourier

calculations, is a transient array form necesarry. The following conditions

here must be fullfilled:

1) start index IS=0

2) end index IE>=16 and power of 2

3) equidistant x-values

At this array form exist IE+1 values.

*****************************************************************************

*            8. Header string, format types and extension                   *

*****************************************************************************

The header-string in the header contains the data versions and the data format:

7- 8: data version DIV 10 at measuring time

9- 11: data version at save time

13-20: data format string

13: main type of format

14: measuring type of format

15: other type of format

16: weighting function of format

17: origin type

18: store type

20: simulation type

The first extension character depends on the main type, the second one normally

on the mesuring type.

main type  1. extension

1       X      --> file from edit plot program without window (ASCII)

2       K      --> file from C/V and I/V-curves

3       Y      --> file with transient data

4       P      --> file with isothermal data (modul 4)

5       T      --> file with tempscan data, constant period width

5       V      --> file with tempscan data, variable period width

6       A      --> file with Arrhenius data (maximum evaluation, ....)

10       Q      --> file for equilibrium measurements (C/T, TSC)

11       0      --> file with local buffer data

12       F      --> file with data from presentation plot program

13       B      --> file with bitmap or box for presentation program

18       I      --> file with user initialization data (ASCII)

meas type  2. extension -------- main type = 2

1       C      --> C/V-high frequency curve

3       I      --> I/V-curve

4       P      --> C/V-pulse curve

5       P      --> C/V-pulse curve

6       P      --> C/V-pulse curve

8       I      --> GS-I/V-curve for FET

9       D      --> DS-I/V-curve for FET

meas type  2. extension -------- main type = 3

1       E      --> emission transient

3       U      --> underground transient without pulse

5       P      --> pulse emission transient

6       C      --> capture transient

meas type  2. extension -------- main type = 4

1       P      --> variation of pulse voltage

2       U      --> variation of reverse bias

3       H      --> variation of pulse voltage and reverse bias

4       F      --> variation of pulse width with fix steps

5       L      --> logarithmic variation of pulse width

6       W      --> variation of period width (frequency scan, ITS)

7       D      --> variation of drain-source voltage

8       D      --> variation of second voltage

9       O      --> variation of optical wave length

10       I      --> variation of optical intensity

meas type  2. extension -------- main type = 5

1       -      --> variable period width

2       -      --> Fourier data and 3 Dlts-signals

3       -      --> Fourier data and 6 Dlts-signals

4       -      --> Fourier data without Dlts-signals

8       -      --> Fourier data and private user correlator signals

meas type  2. extension -------- main type = 6

1       C      --> data from transient capture measurement

3       E      --> data from transient emission measurement

4       F      --> data from pulse width evaluation (capture)

5       W      --> data from period width tempscan maximum evaluation (ITS)

6       T      --> data from tempscan maximum evaluation

8       Z      --> data from Zerbst evaluation

9       O      --> data from period width other maximum evaluation (ITS)

meas type  2. extension -------- main type = 10

1       R      --> measure at UR (for CR(T) or TSC/TSCAP)

2       F      --> measure at UR, but before 1s at UP (for TSC,TSCAP)

3       P      --> measure at UP

11       C      --> from C/V curves

13       I      --> from I/V curves

origin type  --- main type = 1

0          --> unknown

2          --> CV/IV

3          --> transient, start index = 0

4          --> isothermal

5          --> tempscan

6          --> Arrhenius

30          --> as transient, start index = 0

31          --> spectrum

32          --> transient with start index = 1

origin type  --- main type = 2

0          --> normal

1          --> loop of various drain source voltages

2          --> loop of various gate source voltages

5          --> from tempscan

origin type  --- main type = 3

0          --> normal

1          --> tau correction

4          --> from isothermal

5          --> from tempscan

origin type  --- main type = 4, meas type = 6

0          --> only one file

1-5,7-10   --> second variation loop as meas type

6          --> tempscan variation loop

16          --> tempscan variation loop with 2 pulse width, large tp

26          --> tempscan variation loop with 2 pulse width, small tp

origin type  --- main type = 5

0          --> normal

3          --> from transient module

origin type  --- main type = 6, meas type = 9

0          --> no meaning

1-5,7-10   --> from period width maximum evaluation as meas type at

main type = 4, origin type = 4

other type   --- main type = 3, original type = 4

0          --> no meaning

1-10       --> variation of parameter as meas type at main type = 4

other type   --- main type = 4, meas type = 6

1-7        --> mode for variation of period width

store type

0          --> normal

1          --> difference

2          --> file difference

4          --> arrays restored

6          --> 1 point subtracted

simulation type

0          --> no simulation

1          --> simulation

2          --> modifycation

The format type 4 (weighting function) is as the measuring type at main type 5.

*****************************************************************************

*                    Automatic subdirectories                               *

*****************************************************************************

If in the tempscan messurement modul transients, C/V- or I/V-curves, or in

the isothermal program modul transients will be automatically saved, then

a subdirectory will be created with the file name of the tempscan resp.

isothermal file (without extension). The name of the first transient for

the X. file of the constant period width is T001.YX_. The third character of

the transient extension is the same as in the tempscan name. In the following

means nnn the number of transient, C/V or I/V file and z the third character

of extension:

1) Tnnn.Yxz for transient nnn of the x. file of constant period (tempscan)

2) Vnnn.Yxz for transient nnn of the x. file of variable period (tempscan)

3) Tnnn.KCz for the C/V-curve nnn (tempscan)

4) Tnnn.KIz for the I/V-curve nnn (tempscan)

5) Pnnn.YEz for transient nnn at variation of pulse voltage (isothermal)

6) Unnn.YEz at variation of reverse bias (isothermal)

7) Hnnn.YEz at variation of pulse voltage and reverse bias (isothermal)

8) Fnnn.YEz at variation of pulse width in fix steps (isothermal)

9) Lnnn.YEz at logarithmic variation of pulse width (isothermal)

10) Wnnn.YEz at variation of period width (isothermal)

For example, if you save transients in the tempscan modul and your file name is

XYZ.T1A, then the subdirectoy XYZ will be created and all transients will be

saved there. The name of first transient of first file of constant period width

is T001.Y1A.

*****************************************************************************

*                    User initialization files                              *

*****************************************************************************

The user initialization files in the subdirectory \DLTS\DLTFS\INIT are

descriped in the main initialization file CMD.IZA in this subdirectory.